STEP 1

RETUNING A PACIFIC SCEINTIFIC SC450 SERIES MOTOR CONTROLLER

- DO NOT TURN THE VEL.OFFSET POTENTIOMETER unless you are instructed by the a FACTORY Technician

- Current Limit should full clockwise. (100%)

- Command Gain should be full clockwise. (100%)

- Turn the potentiometer on the drive (in the cabinet) labeled Vel. Loop Gain clockwise until the feed begins to oscillate (BUZZ).

- Then turn the Vel. Loop Gain potentiometer counter clockwise until the motor stabilizes then continue 1.5 turns counter clockwise. And your feed will now be tuned.

--------------------------------------------------------------------------------------------

STEP 2

Instructions for Zeroing VEL Offset

- READ ALL DIRECTIONS BEFOR DOING ANYTHING!!!!!!!!!!!!!!!!!!!!!!!

- MAPLE 450 terminal enter the “O” mode and type PGAI enter

- Write the number down that PGAI is set to because you will need to reset this value to ZERO then back to the original number when adjustment is complete.

- Once this parameter is set to 0 the motor should be still however it may move.

- If the motor is moving then ZERO OUT the MOTION with the Vel Offset potentiometer.

- Now if the motor suddenly stops after it was moving DO NOT BE ALARMED.  What happened was the drive built up too much following error and disabled automatically.

- If this happens simply cycle the power and be prepared to adjust the Offset potentiometer on POWER UP.

- Continue doing this until you are able to power up the unit without the motor turning.

- Now you can go back and set PGAI to the original value.