Modification Instructions for Reversing

the Motor Operating Direction on Feeds

Equipped with 450 Series Controllers

All modifications take place within the control cabinet and only involve exchanging several wires.

CAUTION! BEFORE STARTING WORK ENSURE POWER IS DISCONNECTED FROM CONTROL CABINET.

Step 1: locate wires #38 and #39 on main terminal strip. Reverse them.

Step 2: locate wires #25 and #27 on main terminal strip. Reverse them. On feeds produced after 1993 these wires must be swapped directly on the servo drive. The connector is an IDC type. If you do not have the proper tool for this connector call the factory for one!

Step 3: locate connector J54. It is on the bottom of the servo drive controller

slightly under the screen, closest to the back of the controller. Carefully remove plug. Reverse the green/black  and  red/black pairs.(swap the red with the green. then swap the black heat shrunk to the red with the black heat shrunk to the green.)  Replace the plug.

CAUTION! BEFORE RESTORING POWER TO FEED REMOVE ALL MATERIAL FROM FEED ROLLS!

Any questions can be answered by calling (401) 781-3904  (PRF SERVICE DEPT.)