LC3 programming  manual

Waddington Electronics Inc.

1 -PB- PUSHBUTTON

Applies Electrical Power to the Console.

2-PBL- LIGHTED PALMBUTTON

Emergency stop.

NOTE: Always restart in "MANUAL" after an Emergency Stop.

1-SS-THREE POSITION SELECTOR SWITCH

Selects Automatic, Manual, or jog to length (JTL operation)

3-PB-"INDEX" PUSHBUTTON

When 1SS is in "man" position, pushing index feeds out the selected index length.  When 1SS is in the "JTL" position, pushing index will enable the jog forward and reverse push buttons to jog to the next length.

Index must be pushed each time you want to jog to length to the next progression.

4-PB-"JOG FORWARD" PUSHBUTTON (REMOTE)

When 1SS is in "man " position, pushing jog forward results in continuos feeding until released.  When 1SS is in "JTL" position and index has been pushed, jog forward will feed forward only as far as the selected index length at jog speed.

This mode is used for threading progressive dies.

5-PB-"JOG REVERSE" PUSHBUTTON (REMOTE)

When 1SS is in "man" position, pushing jog reverse results in continuos reverse feeding until released. When 1SS is in "JTL" position and index has been pushed, in jog to length mode reverse jogging can only be accomplished after the stock has been jogged forward from the original position.  In jog to length mode jog buttons  only operate within the currently programmed index length.

This mode is used for threading progressive dies.

1-PPB-"CANCEL MOVE" PALMBUTTON

When this pushbutton is depressed, the  feeding will immediately stop; when the pushbutton is pulled out the feeding will continue. The feeding is reset if the feed is jogged before pulling out the pushbutton.

4-SS- THREE POSITION SELECTOR SWITCH

Selects between automatic pilot release, closed rolls, and or open rolls.

2-SS- TWO POSITION SELECTOR SWITCH

Sets anti-backup roll release option on or off (open or closed).

SERIAL INTERFACE TERMINAL

The Alphanumeric terminal has forty two keys, including 26 letters, a numeric keypad, and an enter key.  All data is entered through this terminal and stored in non volatile memory.

Data can be entered by a remote computer or any other terminal with a RS232 Serial Interface

Full Duplex

Baud Rate:9600

Parity: none

Data Wrd:10 bit (8 data, 1 stop, 1 start)

MODES OF OPERATION

There are two modes of operation, Run and User.  The system defaults to the Run mode upon power up.  In this mode, the system will jog or index  upon command.  Data can be interrogated but not changed  in this mode.  Data is interrogated or changed in the user mode.   In this mode, the  system will not jog or index; power is removed  from the motor.  Modes can be changed by entering the appropriate R or U key followed an ENTER.

PROGRAMMABLE PARAMETERS

Parameter                             Abbreviation                            Range

Index Distance                          IND1                                 625.00-+  in.

Acceleration Rate                     ACC1                                 10  -100

Deceleration Rate                     DEC1                                  10   100

Run Speed                                SPD1                                  4700 in/min

Registration Speed                  REG132                               3100 in.min

Index  Complete Window       ICW                                    001  - .600 rev.

Following Error Window        FEW1                                  01 60 rev.

Distance From End                 END1                                  .0064  5 rev.

PARAMETER DEFINITION

Index Distance:

This is the desired length to be indexed. Values are entered in inches only and can be positive or negative. (ie. 5 1/8 inch is entered as 5.125 and -2 inch is entered as -2 )

Acceleration Rate:

This parameter specifies the rate at which the feed accelerates to the programmed Run Speed. A value of 100 corresponds to the maximum or 100% acceleration capability, while 10 is the minimum allowable acceleration.

Deceleration Rate:

This parameter specifies the rate at which the feed decelerates to zero or Registration speed. A value of 100 corresponds to the maximum or 100% deceleration capability, while 10 is the minimum allowable deceleration.

Run Speed:

This parameter specifies the maximum speed attained when indexing. The values are entered in inches per minute.

Registration Speed:

This parameter, used when an overdraw is desired, specifies the speed at which the overdraw is made. There are three types of indexing operation; Index, Index Distance from Registration Mark, and Index to Registration Mark. The Index simply indexes to programmed distance and ignores the Registration Mark input. The Index Distance from Registration Mark accelerates to run speed and runs at run speed until the Registration Mark input goes from high to low, then indexes the programmed distance. The Index to Registration Mark indexes the programmed distance then runs at registration speed until the Registration Mark input goes from high to low and then stops. The Registration Mark operation is only enabled by the manufacturer upon request.

Index Complete Window:

This parameter sets the window size inside which an Index Complete signal will be given at the end of the index. The value programmed is + or - around the final position. It is best to leave this at factory setting.

Following Error Window:

This is the maximum error allowed between the actual and command position before the Following Error output is activated. It is best to leave this at factory setting.

Distance From End:

This parameter specifies the distance from the end of a move at which point the Distance Flag Reached output will be activated. It is best to leave this at factory setting.

Typical Job Set Up

Step  1: Turn selector switch to manual.

Step  2: Depress START button.

Step  3: Go to user mode . Note: (ENTER) means press the ENTER key

TYPE   (ENTER)

Display Responds:U:

Step  4: To enter Length

TYPE  I N D 1 (ENTER)

Display RespondsU:IND1 = 0.500

TYPE  C (ENTER)

Display RespondsU:IND1 =

TYPE  3 . 2 5 (ENTER)

The length is now 3.25 inches

Step  5: To enter Acceleration

TYPE  A C C 1 (ENTER)

Display RespondsU:ACC1 = 30.00

TYPE  C   (ENTER)

Display RespondsU:ACC1 =

TYPE  4 0   (ENTER)

The acceleration is now 40%

Step  6: To enter Deceleration

TYPE  D E C 1   (ENTER)

Display RespondsU:DEC1 = 30.00

TYPE  C (ENTER)

Display RespondsU:DEC1 =

TYPE  4 0 (ENTER)

The Deceleration is now 40%

Step  7: To enter Run Speed

TYPE S P D 1 (ENTER)

Display RespondsU:SPD1 = 900.00

TYPE C (ENTER)

Display RespondsU:SPD1 =

TYPE 4 0 0 (ENTER)

Speed is now set to 400 rpm

Step 8: Switch to run mode

TYPE R (ENTER)

Display responds R:

Step  9: Turn selector switch to AUTO.

Run Mode

All data entered can be interrogated in this mode.

TYPE R (ENTER)

Display RespondsR:

TYPE I N D 1  (ENTER)

Display responds (stored length)

Similarly, acceleration, deceleration, and speed can be interrogated in the Run mode.

The display will respond "UNRECOGNIZED ABBREVIATION" if an unrecognized command is entered. If this happens, try again.

Lc3 programing manual.doc