When you change your engineering unit (ENGU), several other parameters will reset along with it.  These parameters must be changed to the settings that were originally set with the feed.  You may access the parameters as follows:

- Press “O”,  then enter.

- The screen will read “LOCK=”.  Type in then number “4800” and press enter.

- Type in ENGU and press enter.

- The screen will give you the current value.  Press “C” and press enter.

- Type in the new ENGU value and press enter.

- If entered correctly, the screen will go blank for a few seconds, and then now read “ENGU=X”, with “x” being the new value.  If not, repeat steps 3-5.

- Now repeat steps 3-6 for each of the following parameters:

- (Note: Use the same steps to write down these parameters

- prior to changing the ENGU)

| - END1 | - SPD1 |
| --- | --- |
| - PGAI | - JOGH |
| - ALMT | - JOGL |
| - DLMT | - IND1 |
| - ACC1 |  |

When you are through changing your parameters simply type “R” or “U” to return to Run or User mode.  Caution: Do not enter “O” mode for any other purpose unless authorized by a Waddington Electronics technician so as to avoid changing other parameters.

Use the following space to record your parameters prior to changing them.

| New ENGU: | ACC1: (As per customer’s job) |
| --- | --- |
| END1: .01 (will register as .0099999) | SPD1: (As per customer’s job) |
| PGAI: | JOGH: (As per customer’s job) |
| ALMT: | JOGL: (As per customer’s job) |
| DLMT: | IND1: (As per customer’s job) |

If the parameters are provided by Waddington Electronics, all you need to do is confirm the data matches what is in the feed and make the necessary changes.