STEP 1

RETUNING A PACIFIC SCEINTIFIC SC450 SERIES MOTOR CONTROLLER

- DO NOT TURN THE VEL.OFFSET POTENTIOMETER unless you are instructed by the a FACTORY Technician

- Current Limit should full clockwise. (100%)

- Command Gain should be full clockwise. (100%)

- Turn the potentiometer on the drive (in the cabinet) labeled Vel. Loop Gain clockwise until the feed begins to oscillate (BUZZ).

- Then turn the Vel. Loop Gain potentiometer counter clockwise until the motor stabilizes then continue 1.5 turns counter clockwise. And your feed will now be tuned.