Instructions for reprogramming Maple 450B with 450 drive series drive terminal.

1.	In order to get into the program mode it will be necessary to turn off the power on the feed control console. Turn the power on to the feed while holding down the “S” key on the terminal. Continue to hold the key down for 5 seconds after the terminal beeps. You are now in the program mode.

2.	The values in your terminal must exactly match those listed below. Change the values in the terminal by pressing the “9” key until it matches the value on this list. Then press “ENTER” and the next value will be displayed. At the end of the list you will be asked if all values are correct and to press enter to save settings.(note: most of these values will not have to be changed)

LCD VIEWING ANGLE............................................0

SERIAL MODE..........................................................RS-232

# BAUD RATE..............................................................9600

PARITY......................................................................NONE

DATA/STOP BITS......................................................8/1

HANDSHAKING........................................................NONE

TURN AROUND.........................................................NO DELAY

OP MODE...................................................................INTERACTIVE

# LINE TERMINATOR.................................................CRLF

LOCAL ECHO............................................................ DISABLED

DELAYED LF.............................................................DISABLED

TAB WIDTH...............................................................1 SPACE

CURSOR.....................................................................UNDERLINE

KEYBOARD...............................................................ENABLED

LOWER CASE............................................................DISABLED

KEY CLICK................................................................ENABLED

LOCAL SETUP...........................................................ENABLED

SETUP CORRECT?....................................................YES

SAVE SETUP..............................................................YES