Instructions for Zeroing VEL Offset

- READ ALL DIRECTIONS BEFOR DOING ANYTHING!!!!!!!!!!!!!!!!!!!!!!!

- Type O (enter).

- LOCK=4800 (enter)

- PGAI (enter) WRITE THIS VALUE DOWN we will be setting it to zero in a moment.

- C (enter)

- 0(enter)

- As soon as you press (enter) the motor should be still however it may move.

- If the motor is moving then ZERO OUT the MOTION with the Vel Offset potentiometer. As you turn the potentiometer closer to the zero point the motion of the motor will slow and then it will stop when you get to the zero point if you go past the zero point motion will change direction. Just keep adjusting until the motor doesn’t move.

- Now if the motor suddenly stops after it was moving DO NOT BE ALARMED.  What happened was the drive built up too much following error and disabled automatically.

- If this happens simply cycle the power and be prepared to adjust the Offset potentiometer on POWER UP.

- Continue doing this until you are able to power up the unit without the motor turning.

- Now you can go back and set PGAI to the original value.