Instructions for programming a 750 drive from scratch

Note: Switching RS-232 cables or communications programs should be done only with the power off or the 750 may fault and force a reload on the entire program.

Connect a pc running 750 programming to the feed serial port

power up system

Compile and run the program

When program in running press <F4> to get output window

Stop program with control C

Set NV variables using the set variables command in the compile pull down menu

File     Edit   Search    ¦Compile¦   Options             F1 - Help

+----------------------------+

+- I:\PROGRAM\750\DISK\NON¦ Compile Program            ¦nes In File 00001 -+

¦                         +----------------------------¦                   -

¦                         ¦ Run Program         Ctrl+R ¦                   ¦

¦                         ¦ Reset Program        Alt+R ¦                   ¦

¦                         ¦ Continue Program        F5 ¦                   ¦

¦                         ¦ Stop Program        Ctrl+C ¦                   ¦

¦                         +----------------------------¦                   ¦

¦                         ¦ Output Window           F4 ¦                   ¦

¦                         +----------------------------¦                   ¦

¦                         ¦ Set BreakPoint          F9 ¦                   ¦

¦                         ¦ Clear All BrkPts           ¦                   ¦

¦                         +----------------------------¦                   ¦

¦                         ¦ Variables...               ¦                   ¦

¦                         ¦ Real Time Function Call... ¦                   ¦

¦                         +----------------------------+                   ¦

¦                                                                          ¦

¦                                                                          ¦

¦                                                                          ¦

¦                                                                          ¦

¦

+¦¦¦ Axis 255 ¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦ Line 0001 Col 01 ¦¦¦+

Press F10 for Dropdown Menus.

PACIFIC SCIENTIFIC  750 Fast Compiler/PACLAN/CAM     Version 2.11¦¦¦10:37:57 AM

Type variable to be changed on the top line.

then arrow down to bottom line to enter new value.

File     Edit   Search    ¦Compile¦   Options             F1 - Help

+----------------------------+

+- I:\PROGRAM\750\DISK\NON¦ Compile Program            ¦nes In File 00001 -+

¦                         +----------------------------¦                   -

¦                         ¦ Run Program         Ctrl+R ¦                   ¦

¦                         ¦ Reset Program        Alt+R ¦                   ¦

¦                +---------------- Variables -----------------+            ¦

¦                ¦                                            ¦            ¦

¦                ¦+------- Enter A Variable Name ------------+¦            ¦

¦                ¦¦                                          ¦¦            ¦

¦                ¦+------------------------------------------+¦            ¦

¦                ¦+----------- Current Value ----------------+¦            ¦

¦                ¦¦                                          ¦¦            ¦

¦                ¦+------------------------------------------+¦            ¦

¦                ¦+----------- Enter a Value ----------------+¦            ¦

¦                ¦¦                                          ¦¦            ¦

¦                ¦+------------------------------------------+¦            ¦

¦                +--------------------------------------------+            ¦

¦                                                                          ¦

¦                                                                          ¦

¦                                                                          ¦

¦

+¦¦¦ Axis 255 ¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦¦ Line 0001 Col 01 ¦¦¦+

Press F10 for Dropdown Menus.

PACIFIC SCIENTIFIC  750 Fast Compiler/PACLAN/CAM     Version 2.11¦¦¦10:40:07 AM

Set parm(5) = 13000 ----machine will give you a number for this the 1st time you 				try to home it.

Set engr  =  9.420467   ---	use your number here

Set rat = 3		--- 	use pulley ratio here

LEV = 98

OTHER VARIABLES MAY HAVE TO BE SET FROM THE dim PART OF THE MAIN PROGRAM

Power down

Shut down 750 program

Start up procom program

Press alt D

hit enter on 750

wait 5 seconds

hit escape

power up 750

When machine asks for home type wad01

machine responds OK

type comps

press page up

procom responds

+--¦ Upload Protocols ¦-------------------------------------------------

¦

¦ 1) XMODEM        5) TELINK         9) WXMODEM       13) YMODEM-G BATCH

¦ 2) KERMIT        6) MODEM7        10) IMODEM        14) zmodemu

¦ 3) YMODEM        7) SEALINK       11) YMODEM-G      15) EXTERN 2

¦ 4) ASCII         8) COMPUSERVE B  12) YMODEM BATCH  16) EXTERN 3

¦

¦ Your Selection:     (or press ENTER for ASCII)

select 4

procom responds

+---¦ ASCII UPLOAD ¦--------------------------------------------------+

¦                                                                     ¦

¦ Please enter filename:                                              ¦

¦                                                                     ¦

+---------------------------------------------------------------------+

tell procom where the xxxxxcmp.txt file is

example a:\14156cmp.txt

NOTE: The first 5 digits are the serial number for your machine.

hit enter

procom will then upload comps file to the control

you will see the numbers scrolling by

up-load will then terminate

to see uploaded comps type dump

750 will then dump out the uploaded comps

type quit.

power down 750

restart.

program should run.

Refer to the SC750 drive mannuals for assistance to perform the following.

A first attempt to load should be from the complie dropdown

menu. First open the .bas file from the file drop down menu then

from the complie dropdown menu stop the control then run the control.

This will complie and download the program to the control.

SYSTEM VARIABLES

These values are set through the PAC-SCi software. The compile drop

down menu must be activated and from there select stop program then

variables. The following variables must be edited if the drive is

loaded from an unconfigured state.

autostart = 1

parm(1) = 0.9500

parm(2) = 5.000

parm(3) = 10.000

parm(4) = 27

parm(5) = 47625

parm(6) = 55.00

parm(7) = 150.00

engr = 9.422350

rat = 3.0

lev = 0

PROGRAM VARIABLES

jogl = 100