OPERATION

Feed modes are controlled by the AUTO/MAN/JTL selector switch.

When AUTO is selected, the feed will index whenever an index signal is

sensed at terminal 14. Programming is not possible in this mode. When

JTL is selected the feed will jog the programmed index amount by

pressing the index pb then the jog forward pb for forward motion or

jog reverse for reverse motion. In the MAN position the feed will jog

as long as the jog fwd or jog rev pbs are pressed.If index is pressed

the feed will index the programmed amount in the forward direction only.

When the machine is powered up, the display will tell the operator to

change modes, if it was powered up in AUTO.

HOMING AND RE-HOMING

To make a job active press j=(job number) then the enter key.

The display will show JOB # = (job number). This means that (job number) is now

the active job.

PROGRAMMING

Two methods of variable entry are available. One method prompts

the operator for data for all required variables. The other method is to

simply enter the variable name with the value. For example to use the

prompted method press the "M" key followed by the enter key. This will

begin the prompted method. For ease of explanation the enter key will be

shown as <enter>, display text will be upper case and default values will

be displayed as [12]. The following sequence will go through all steps

for the prompted method of data entry following the M input.

ENTER THE JOB NUMBER [2]

1 <enter>

JOB # 1

ENTER THE NUMBER OF PROFILES FOR THIS

JOB [10]

2 <enter>

# OF PROFILES 2

ENTER INDEX LENGTH IN INCHES

I 1 = [1.005]

0.025 <enter>

I 1 = 0.025

ENTER THE VELOCITY IN IPM [100]

500 <enter>

SPD = 500

ENTER THE ACCELERATION 1-100% [75]

50 <enter>

ACC = 50

ENTER THE GAG OUTPUT WORD 1 (0-1)

[11000101]

10000001 <enter>

GAG OUTPUTS FOR PROFILE 1 = 10000001

ENTER PROFILE 1 NUMBER OF REPEATS [25]

1 <enter>

# OF REPEATS FOR PROFILE 1 = 1

ENTER THE INDEX COMPLETE DURATION IN 10

MILLISECOND INCREMENTS [0]

<enter>  (DEFAULT ACCEPTED)

OUTPUT ON FOR 10 MILLISECONDS

ENTER INDEX LENGTH IN INCHES

I 2 = [0.000]

0.25 <enter>

I 2 = 0.25

ENTER THE GAG OUTPUT WORD 2 (0-1)

[00000000]

00010001 <enter>

GAG OUTPUTS FOR PROFILE 2 = 00010001

ENTER PROFILE 2 NUMBER OF REPEATS [0]

3 <enter>

# OF REPEATS FOR PROFILE 2 = 3

Prompts for lengths, gag outputs, and number of repeats will be

repeated for for the total number of profiles. If a value outside of

the valid range is entered, a message will appear on the display to

re-enter the value. The default value displayed may be accepted simply by

pressing the enter key.

The second method is a simple way of entering values. To enter the

the index length as 12.0" type I1=12.0 then press the enter key. An alternate

way is to type I1 12.0 and press the enter key. This method can also be used to

edit values in an existing job. For example to edit the index length in profile

3 of job 2 from 1.5 to 1.45 the following must be done.

j=2 <enter>	to activate job 2 (not required if job 2 already active)

JOB # = 2	display shows active job

i3=1.45 <enter> 	to edit ind3

I 3 = 1.45		diplay shows value

Variable JOGL is not prompted and may be changed by typing JOGL=100

followed by the enter key. The following are a few example entries.

I1=3.92699 ENTER	 or	 I1 3.92699 ENTER

S1=1200 ENTER			 S1 1200 ENTER

A1=50 ENTER			 A1 50 ENTER

D3=0 ENTER			 D3 0 ENTER

G2=11001000			 G2 11001000

JOGL=100 ENTER			JOGL 100 ENTER

VARIABLES

QUICK REFERENCE

NAME	UNITS	USED	PROMPTED	FUNCTION

IND	INCHES	1/PROF	YES		INDEX LENGTHS

SPD	IPM	1/JOB	YES		INDEX SPEEDS

ACC	%	1/JOB	YES		ACCELERATIONS

DUR	10MSEC	1/JOB	YES		INDEX COMPLETE DURATION TIME

GAG	BITS	1/PROF	YES		GAG OUTPUT WORD 10011010 (1=ON, 0=OFF)

REP	INTEGER 1/PROF	YES		NUMBER OF REPITITIONS

JOGL	IPM	1/ALL	NO		JOG SPEED

EXPLANATION

JOB

The job number is the way that the control identifies jobs. Any non-

zero integer number between 1 and 2 is valid.

# OF PROFILES

The number of profiles is the total number of different lengths to be

run in sequence to complete a job. When the current profile number reaches the

total number of profiles the job is complete and the job starts over. Any non-

zero integer value between 1 and 128 is valid.

IND

Ind is the index length in inches. Each profile must have a non-zero

real value. Valid values are any value between +/-1200.0 and +/-0.001.

SPD

Spd is the speed or feedrate which the feed will use to move material

through for the entire job. Any non-zero integer value between 1 and 9425 is

valid. Units are inches per minute.

ACC

Acc is the acceleration rate which the feed will use to acquire its

running speed. This parameter is a percentage. 100% is equal to 1832.6 i/s/s.

Any integer between 1 and 100 is valid.

GAG

The gag word is a string of 1 and 0 to indicate which outputs will be

on or off for the current profile. There must be 8 digits in each gag word.

1 indiactes that the output will be on and 0 indicates it will be off. For

example for a gag word of 10000100 the eight outputs would react as follows:

gag0 = 0 = off

gag1 = 0 = off

gag2 = 1 = on

gag3 = 0 = off

gag4 = 0 = off

gag5 = 0 = off

gag6 = 0 = off

gag7 = 1 = on

REP

Rep is the number of repititions of a profile. Any integer between 1

and 65535 is valid.

DUR

Dur is the time that the index complete relay will be energized. Any

integer value between 0 and 3600 is valid. The value represents 10 millisecond

increments. If a value of 3 was entered then the relay will be energized for

30 milliseconds. If a value of 0 was entered the relay will be energized as

long as the feed is not moving.

FEATURES

To display jobs stored in memory, perform the following steps.

L <enter>

VALID JOBS 1 , 2

In order to run, edit, or perform any other function to a job, the job

must be active. To make a job active type j=(job number) then the enter key.

The display will show JOB # = (job number). This means that (job number) is now

the active job.

To delete a job from memory load another job in its place. An alternate

method is to perform the following steps.

DELETING JOBS

ADDING JOBS

ADDING PROFILE	AUTO PROMPTING

DELETING PROFILES

Jobs may be up and down loaded to a computer but the data must be

formatted as shown below. The computer can be connected to the 9 pin d-shell

connector mounted on the side of the operator's station.

PINS		FUNCTION

1		SHIELD

2		TXD

3		RXD

5		COMMON

The control is set up for rs232 serial communication at 9600 baud,1 start bit,

1 stop bit, and 8 data bits.To download to a computer, the computer will be

used as a terminal. Software such as pcplus is very useful. First activate the

desired job then type dn <enter>. The control will feed data in ascii format.

After all data has been transfered, COMPLETE will be transmitted followed by an

EOT control character.To up load data type up <enter>. Data should be transmitted

in ascii format in the following line format.

J=(job number 1 or 2)<cr>

N(job number)=(1 to 128)<cr>

S(job number)=(1 to 9425)<cr>

A(job number)=(1 to 100)<cr>

D(job number)=(0 to 3600)<cr>

I1=(real value)<cr>

G1=(string of 8)<cr>

R1=(integer value)<cr>

The last 3 lines must be repeated until all profile data is transmitted. The

control will be looking for an EOT control character to complete the task.

RUNNING AND CYCLE SEQUENCE