Waddington Electronics Inc.

25 Webb St. Cranston R.I.02920

(401)-781-3904 Fax (401)-781-1650

# Waddington Electronics Inc.

# IMEC 750 PROGRAMMING 1

In order to program the feed, the AUTO/MAN/JTL selector must not be in the AUTO position.

All programming must be done in either the MAN or the JTL modes. When the machine is powered up the display will tell the operator to change modes if it was powered up in AUTO.

Two methods of data entry are available.

One method prompts the operator for data for all required variables. The other method is to simply enter the variable name with the value.

For example

To use the prompted method press the "M" key followed by the enter key.

This will begin the prompted method.

The first prompt "ENTER THE INDEX LENGTH IN INCHES" will appear on the display followed by the current value.

If the current value is desired just press the enter key to retain that value.

If another value is desired then enter the value and press the enter key.

If an error is made while entering a value the value must be re-entered

after completion.

If a value outside of the valid range is entered, a message will appear on the display to re-enter the value.

For the index length the valid range of value is 0.005" to 1200.000".

The display will show IND 1 = "VALUE" when a value is entered successfully. The next prompt will be ENTER THE VELOCITY IN IPM followed by the current value.

Again the current value may be retained or a new value entered.

The display will show SPD 1 = "VALUE".

The next prompt will be "ENTER ACCELERATION 1-100%" followed by the actual value.

Here 100% equals 35,000 RPM/SEC.

The next prompt will be "ENTER THE INDEX COMPLETE DURATION IN 10 MILLISECOND INCREMENTS" followed by the current value.

Entering a value of zero will cause the index complete relay to be on whenever the feed is in position (stopped).

A value other than zero will cause the relay to be on only for the programmed duration which begins when the feed reaches its programmed position. The display will show complete when all prompts are complete.

The second method is a simple way of directly entering values.

To enter the index length as 12.0" type I=12.0 then press the enter key.

An alternate way is to type I 12.0 and press the enter key.

See the list of variables below.

Note that variable TYP (acceleration Type) is only entered in this way and is not prompted. This is the only way to enter the TYP variable

Variable JOGL is also not prompted and may be changed by typing JOGL=100 followed by the enter key. The following are a few example entries.

I=3.92699 ENTER 	or		I 3.92699 ENTER

S=1200 ENTER			S 1200 ENTER

A=50 ENTER			A 50 ENTER

T=0 ENTER				T 0 ENTER

D=0 ENTER				D 0 ENTER

JOGL=100 ENTER			JOGL 100 ENTER

VARIABLES

NAME			PROMPTED		FUNCTION

IND			YES			INDEX LENGTHS

SPD			YES			INDEX SPEEDS

ACC			YES			ACCELERATIONS

DUR			YES			INDEX COMPLETE DURATION TIME

TYP			NO			ACCEL. TYPES (0 =CONSTANT,1 =S CURVE)

JOGL			NO			JOG SPEED