REVISION B

OPERATION

Feed modes are controlled by the AUTO/MAN/JTL selector switch.

When AUTO is selected, the feed will index whenever an index signal is sensed at terminal 14. Programming is not possible in this mode. When JTL is selected the feed will jog the programmed index amount by pressing the index pb then the jog forward pb for forward motion or jog reverse for reverse motion. In the MAN position the feed will jog as long as the jog fwd or jog rev pbs are pressed.If index is pressed the feed will index the programmed amount in the forward direction only.When the machine is powered up, the display will tell the operator to change modes, if it was powered up in AUTO.

HOMING AND RE-HOMING

Homing the feed must be done whenever the feed is powered up. If it is desired to home the feed at some other time just press H followed by the enter key.

CHANGING ACTIVE JOBS

To make a job active press j=(job number) then the enter key. The display will show JOB # = (job number). This means that (job number) is now the active job.

PROGRAMMING

Two methods of variable entry are available. One method prompts The operator for data for all required variables. The other method is to simply enter the variable name with the value. For example to use the prompted method press the "M" key followed by the enter key. This will begin the prompted method. For ease of explanation the enter key will be shown as <enter>, display text will be upper case and default values will be displayed as [12]. The following sequence will go through all steps for the prompted method of data entry following the M input.

Numbers shown in [] (Brackets are defaults or current value) to keep default just press enter.

EXAMPLE 1 (WITHOUT FEED ADVISOR)

ENTER THE JOB NUMBER [2]		DISPLAYED ON TERMINAL

1 <enter>					ENTERED AT KEYPAD

JOB # 1						DISPLAYED ON TERMINAL

ENTER THE NUMBER OF PROFILES FOR THIS

JOB [10]						DISPLAYED ON TERMINAL

2 <enter>					ENTERED AT KEYPAD

# OF PROFILES 2					DISPLAYED ON TERMINAL

ENTER INDEX LENGTH IN INCHES

I 1 = [1.005]					DISPLAYED ON TERMINAL

0.025 <enter>				ENTERED AT KEYPAD

I 1 = 0.025						DISPLAYED ON TERMINAL

ENTER THE VELOCITY IN IPM [100]	DISPLAYED ON TERMINAL

500 <enter>					ENTERED AT KEYPAD

SPD = 500						DISPLAYED ON TERMINAL

ENTER THE ACCELERATION 1-100% [75]

50 <enter>

ACC = 50

ENTER THE GAG OUTPUT WORD 1 (1-0)  [00100000]

01000000 <enter>

GAG OUTPUTS FOR PROFILE 1 = 01000000

ENTER PROFILE 1 NUMBER OF REPEATS [25]

1 <enter>

# OF REPEATS FOR PROFILE 1 = 1

ENTER THE INDEX COMPLETE DURATION IN 10

MILLISECOND INCREMENTS [0]

<enter>  (DEFAULT ACCEPTED)

OUTPUT ON FOR 10 MILLISECONDS

ENTER INDEX LENGTH IN INCHES

I 2 = [0.000]

0.25 <enter>

I 2 = 0.25

ENTER THE GAG OUTPUT WORD 2 (BINARY)		[00010000]

00100000 <enter>

GAG OUTPUTS FOR PROFILE 2 = 00100000

ENTER PROFILE 2 NUMBER OF REPEATS [0]

3 <enter>

# OF REPEATS FOR PROFILE 2 = 3

EXAMPLE 2 (WITH FEED ADVISOR)

ENTER THE JOB NUMBER [2]

1 <enter>

JOB # 1

ENTER THE NUMBER OF PROFILES FOR THIS

JOB [10]

2 <enter>

# OF PROFILES 2

ENTER INDEX LENGTH IN INCHES

I 1 = [1.005]

0.025 <enter>

I 1 = 0.025

ENTER Y TO USE THE FEED ADVISOR 		Note: new feature. See back of manual

Y <enter>						Pressing enter without Y will skip advisor

ENTER THE MAXIMUM SPM FOR THE JOB

300 <enter>

ENTER THE FEED OPERATING ANGLE

100 <enter>

ENTER THE GAG OUTPUT WORD 1 (1-0)  [00100000]

01000000 <enter>

GAG OUTPUTS FOR PROFILE 1 = 01000000

ENTER PROFILE 1 NUMBER OF REPEATS [25]

1 <enter>

# OF REPEATS FOR PROFILE 1 = 1

ENTER THE INDEX COMPLETE DURATION IN 10

MILLISECOND INCREMENTS [0]

<enter>  (DEFAULT ACCEPTED)

OUTPUT ON FOR 10 MILLISECONDS

ENTER INDEX LENGTH IN INCHES

I 2 = [0.000]

0.25 <enter>

I 2 = 0.25

ENTER THE GAG OUTPUT WORD 2 (BINARY)		[00010000]

00100000 <enter>

GAG OUTPUTS FOR PROFILE 2 = 00100000

ENTER PROFILE 2 NUMBER OF REPEATS [0]

3 <enter>

# OF REPEATS FOR PROFILE 2 = 3

Prompts for lengths, gag outputs, and number of repeats will be repeated for for the total number of profiles. If a value outside of the valid range is entered, a message will appear on the display to re-enter the value. The default value displayed may be accepted simply by pressing the enter key.

The second method is a simple way of entering values. To enter the the index length as 12.0" type I1=12.0 then press the enter key. An alternate way is to type I1 12.0 and press the enter key. This method can also be used to edit values in an existing job. For example to edit the index length in profile 3 of job 2 from 1.5 to 1.45 the following must be done.

j=2 <enter>		to activate job 2 (not required if job 2 already active)

JOB # = 2		display shows active job

i3=1.45 <enter> 	to edit ind3

I 3 = 1.45		diplay shows value

Variable JOGL is not prompted and may be changed by typing JOGL=100 or some other value followed by the enter key. The following are a few example entries.

I1=3.92699 ENTER	 or	 I1 3.92699 ENTER

S1=1200 ENTER			 S1 1200 ENTER

A1=50 ENTER				 A1 50 ENTER

D3=0 ENTER				 D3 0 ENTER

G2=64 ENTER				 G2 64 ENTER

JOGL=100 ENTER			JOGL 100 ENTER

VARIABLES

QUICK REFERENCE

NAME	UNITS	      USED		PROMPTED	FUNCTION

IND	INCHES	1/PROF	 YES		INDEX LENGTHS

SPD	IPM		1/JOB		 YES		INDEX SPEEDS

ACC 	%		1/JOB		 YES		ACCELERATIONS

DUR	10MSEC	1/JOB		 YES		INDEX COMPLETE DURATION TIME

GAG	BITS		1/PROF	 YES		GAG OUTPUT WORD 10011010 (1=ON, 0=OFF)

REP	INTEGER	1/PROF	 YES		NUMBER OF REPITITIONS

JOGL	IPM		1/ALL		 NO		JOG SPEED

EXPLANATION

JOB

The job number is the way that the control identifies jobs. Any non-

zero integer number between 1 and 25 is valid.

# OF PROFILES

The number of profiles is the total number of different lengths to be

run in sequence to complete a job. When the current profile number reaches the

total number of profiles the job is complete and the job starts over. Any non-

zero integer value between 1 and 9 is valid.

IND

Ind is the index length in inches. Each profile must have a non-zero

real value. Valid values are any value between +/-1200.0 and +/-0.001.

SPD

Spd is the speed or feedrate which the feed will use to move material

through for the entire job. Any non-zero integer value between 1 and 8475 is

valid. Units are inches per minute.

ACC

Acc is the acceleration rate which the feed will use to acquire its

running speed. This parameter is a percentage. 100% is equal to 1832.6 i/s/s.

Any integer between 1 and 100 is valid.

GAG

The gag word is a binary weighted integer on or off for the current profile. For

example for a gag word of 00100000 the eight outputs would react as follows:

gag1 = 0 = off 0

gag2 = 0 = off 0

gag3 = 0 = off 0

gag4 = 0 = off 0

gag5 = 0 = off 0

gag6 = 1 = on  32

gag7 = 0 = off 0

gag8 = 0 = off 0

In this case G1=32

Note: When programming in the prompted mode  a string of 8 zeroes and ones are 	entered. When programming gag words without promts the binary intergers

must be entered.

PROMTED				NON-PROMPTED

00001111				15

10010110				150

REP

Rep is the number of repititions of a profile. Any integer between 1

and 65535 is valid.

DUR

Dur is the time that the index complete relay will be energized. Any

integer value between 0 and 3600 is valid. The value represents 10 millisecond

increments. If a value of 3 was entered then the relay will be energized for

30 milliseconds. If a value of 0 was entered the relay will be energized as

long as the feed is not moving.

FEATURES

To display jobs stored in memory, perform the following steps.

L <enter>

VALID JOBS 1, 2, 3, 8

In order to run, edit, or perform any other function to a job, the job must be active. To make a job active type j=(job number) then the enter key. The display will show JOB # = (job number). This means that (job number) is now the active job.

To delete a job from memory load another job in its place.

To copy a job press CO<enter>. The display will prompt for the job to be copied then

prompt for the new job. The display will show COMPLETE when the job is copied.

Jobs may be up and down loaded to a computer but the data must be formatted as shown below. The computer can be connected to the 25 pin d-shell connector mounted on the side of the operator's station.

PINS		FUNCTION

1		SHIELD

2		TXD

3		RXD

10		COMMON

The control is set up for rs232 serial communication at 9600 baud,1 start bit, 1 stop bit, and 8 data bits.

To download to a computer, the computer will be used as a terminal. Software such as pcplus is very useful. First activate the desired job then type DN <enter>.

The control will feed data in ascii format. After all data has been transfered, COMPLETE will be transmitted followed by an EOT control character (Ctrl D). To up load data type UP <enter>. Data should be transmitted in ascii format in the following line format.

J=(job number 1 to 25)<cr>

N(job number)=(1 to 9)<cr>

S(job number)=(1 to 9425)<cr>

A(job number)=(1 to 100)<cr>

D(job number)=(0 to 3600)<cr>

I1=(real value)<cr>

G1=(integer value)<cr>

R1=(integer value)<cr>

NOTE: When transfering data or non-prompted programming gag words must be in binary 	format.

00000000		0

00000001		1

00000010		2

00000100		4

00001000		8

00010000		16

00100000		32

01000000		64

10000000		128

The last 3 lines must be repeated until all profile data is transmitted. The

control will be looking for an EOT control character to complete the task.

Example file format

A2=40

D2=1

S2=500

T2=0

N2=3

I1=0.381

G1=32

R1=1

I2=0.0325

G2=2

R2=1

I3=0.5433

G3=4

R3=1

<eot>

The Above example will set up job number 2 with an accelration value of 40 %, Index complete relay will turn on for 10 ms,  Max speed will be 500 IPM,  Acceltation type will be trapozoidal, and number of profiles will be 3.

When the cycle  is started profile 1 will be the active profile. Using the above example, the feed will increment 0.381" with gag 5 active. Since the number of repititions for profile 1 is 1 the profile number will be incremented to 2 for the next index. The feed will index 0.0325" with gag 1 active and increment the profile number to 3. The values in profile 3 will be used and the part will be complete. If the part counter preset was set to 1 then the press stop output will energized and if wired to the press stop circuit, the press will stop. If the parts count preset was set to 10 then the feed will run in the same sequence as described above for 9 more times.

The parts counter may be reset by pressing PR followed by the enter key.

The parts count preset may be changed by pressing PC=(num) followed by the enter key. (num) may be any desired value between 1 and 999999.

If for any reason a particular profile must be run out of normal sequence do the following steps. Be sure feed is not in auto mode. Press P=(desired profile number)<cr>. The next index will be the amount programmed in the selected profile and continue through the program from that point.

NEW FEATURE

A feed advisor feature has been added to the software. This feature will calculate the required speed and acceleration for the specific job. The operator is required to answer the feed advisor prompt with the Y key and press enter. This will activate

the feed advisor software. The control will prompt for the strokes per minute and

the feed angle. This information will be used to calculate the feed’s speed and acceleration.